VIT  -  Computer Networks Project

REVIEW 2 - DESIGN & IMPLEMENTATION

Assessment 9 component  |  Upload on VTOP on or before 9 October 2026

Adaptive Multi-Error FEC Link Layer for High-Interference Environments

An extension of the approved project "Fast-Lane FEC for Automated Toll Booths"

Team: [Your Name] [Reg. No.]  |  [Teammate Name] [Reg. No.]

Guide: [Guide Name, Designation]

Progress status: version v0.2 (feedback, ARQ/HARQ, adaptive controller, dashboard)

Source code: github.com/PavitranVC/Adaptive-FEC-Link

# Contents

1. Introduction and Problem Statement

2. System Architecture

3. Detailed Design

4. Methodology / Algorithms

5. Database / UML / DFD

6. Module Development

7. Working Prototype

8. Implementation Progress

9. Test Cases

10. Initial Results

11. Individual Contribution

12. Plan for Review 3 and References

# 1. Introduction and Problem Statement

Think about a car going through an automated toll booth. The booth has to read a 96-bit RFID tag ID in the short time the car is in range, and the radio link it uses is noisy. Some bits get flipped at random, and now and then a passing truck or another reader wipes out a whole run of bits. If a read fails, the toll is missed or the car has to stop. Asking the car to send the ID again is not a great answer either, because the car may already be gone by the time the request arrives. So the receiver should fix the errors itself, using forward error correction (FEC), and a CRC-32 checksum [5] on top makes sure a wrong ID is never accepted by mistake.

Our project grows the approved topic "Fast-Lane FEC for Automated Toll Booths" into an adaptive, multi-error FEC link layer. We simulate it in software as three UDP programs: a vehicle, a noisy channel and a tollgate. To see how the codes behave on very different noise, we also added a second scenario, a hospital with MRI and CT equipment, where interference comes in bursts.

### Objectives

Write a family of error-correcting codes ourselves in C11 without external libraries: Hamming, SECDED, BCH and Reed-Solomon [1], [2], plus a bit interleaver and a CRC-32 for error detection.

Model the noise with three models (binary symmetric channel, Gilbert-Elliott [3], [4] and single bursts), packaged into two interference profiles.

Build a real three-process UDP link with a feedback path, so that it behaves like a working system and not only a set of calculations.

Compare plain FEC with Stop-and-Wait ARQ, Hybrid ARQ and an adaptive controller that picks a stronger or weaker code based on what the receiver reports.

Measure success rate, silent-wrong rate, goodput, retransmissions and latency, using seeded random numbers so anyone can repeat a run and get the same answer.

# 2. System Architecture

Figure 1. System architecture: three UDP processes, feedback path and shared library.

The vehicle appends a CRC-32 to the 96-bit ID, encodes the result with the chosen code and sends it on UDP port 9000. The channel program flips bits according to the noise model and passes the frame on to port 9001, together with a small extra field that holds the true ID. That field is only used for scoring and the decoder never reads it. The tollgate decodes the frame, checks the CRC and labels the result, then replies on port 9002 with an ACK or NAK and some recent statistics. All three programs link the same library, which means the benchmark and the real UDP programs run exactly the same code.

Table 1. Components

| Component | Responsibility |
| --- | --- |
| bin/vehicle | encode, send, sender state machine (FEC / ARQ / HARQ), adaptive controller |
| bin/channel | noise injection (BSC / GE / burst), ground-truth sidecar, optional header corruption |
| bin/tollgate | decode, CRC-32, classification, feedback, per-frame log and coloured bit map |
| bin/bench | in-process sweeps (no sockets) -> results/*.csv |
| tools/dashboard.py | stdlib-only live web dashboard fed by the tollgate JSON-lines log |

### Frame classification

CORRECT - CRC passes and ID equals the truth.

DETECTED_FAIL - CRC fails or the decoder flags an uncorrectable block; the system knows not to bill.

SILENT_WRONG - CRC passes but the ID is wrong; the dangerous case, measurable only because the channel attaches ground truth that the decoder never reads (checked by a test).

# 3. Detailed Design

## 3.1 Frame format

Figure 2. Frame layout: unprotected header plus FEC(ID96 || CRC32).

The 16-byte header holds the magic number, version, type, code, rs_t, interleave depth, sequence number, bit count, attempt number and strategy. In this version the channel leaves the header alone; that was a deliberate decision (DD-01) to keep the first experiments simple. The payload is 128 bits before coding.

## 3.2 Codes implemented

Table 2. Codes available at this review (LDPC is planned as a stretch for Review 3)

| Code | Rate | Corrects | Notes |
| --- | --- | --- | --- |
| uncoded | 1.0000 | - | baseline |
| Hamming(7,4) | 0.571 | 1 | cheap fallback; syndrome = error position |
| Hamming(15,11) | 0.711 | 1 | higher rate |
| SECDED(8,4) | 0.500 | 1 (+2 detected) | extended Hamming |
| BCH(15,7) | 0.449 | 2 | GF(16), Berlekamp-Massey + Chien |
| BCH(31,16) | 0.516 | 3 | best on random errors (original codes) |
| Reed-Solomon RS(16+2t,16) | 0.889 .. 0.333 | t bytes (default 4) | GF(256), shortened, Forney; bursts |

## 3.3 Interference profiles

Table 3. Interference profiles (modelling assumptions, not field measurements)

| Profile | BSC | Gilbert-Elliott | Burst |
| --- | --- | --- | --- |
| TOLL_PLAZA (primary) | p = 0.01 | p_gb 0.002, p_bg 0.5, e_good 0.005, e_bad 0.5 | L = 4 bits in 20% of frames |
| HOSPITAL_IMAGING (secondary) | p = 0.02 | p_gb 0.005, p_bg 0.08 (mean burst 12.5 bits), e_good 0.002, e_bad 0.5 | L = 16 bits in 50% of frames |

## 3.4 Link strategies

Table 4. Strategies (--strategy)

| Strategy | Behaviour |
| --- | --- |
| fec | one shot, no retransmission |
| arq | Stop-and-Wait, uncoded + CRC, retransmit on timeout/NAK (the strawman) |
| harq | HARQ type I: FEC first, retransmit only when CRC fails (no soft combining) |
| adaptive | vehicle changes the code level from receiver window statistics (W = 32 frames) |

Figure 3. Sender state machine (ARQ/HARQ/adaptive).

## 3.5 Adaptive controller

Figure 4. Adaptive code-ladder controller with hysteresis.

The tollgate keeps a sliding window of the last 32 frames and reports how many failed. If more than 20% failed, the vehicle moves up to a stronger code. Moving down is harder on purpose: it only happens when a binomial model says the cheaper code would fail less than 5% of the time, and a probe back-off stops the controller from bouncing between levels. Our first version checked after every single frame and flipped levels 14 times in one run. Checking once per window fixed that, and we wrote the lesson in the design log.

# 4. Methodology / Algorithm

CRC-32 (IEEE 802.3): the checksum is the remainder of m(x)*x^32 divided by G(x). It can detect errors but never fix them, and a random wrong word gets through with probability of only about 2^-32.

Hamming: the parity-check matrix H has the numbers 1 to n as its columns, so the syndrome of a received word is simply the position of the flipped bit. SECDED adds one overall parity bit so a single error can be told apart from a double error.

BCH: the generator g(x) is the lcm of the minimal polynomials of alpha^1 to alpha^2t. To decode we compute the syndromes S_j = r(alpha^j), use Berlekamp-Massey to find the error-locator polynomial and then a Chien search to find where the errors are.

Reed-Solomon over GF(256): the same steps, but working on whole bytes, plus Forney's formula to get the error values. A burst of up to 8t bits only damages a few bytes, which is why RS survives bursts that wreck bit-level codes.

Block interleaver: bits are written row by row and sent column by column, so a burst of L bits turns into about L/d errors per codeword. This helps bit-level codes but, surprisingly, hurts RS: on the hospital profile RS t=4 dropped from 83.4% to 59.8% with depth 8.

Noise: BSC flips bits independently; Gilbert-Elliott is a two-state Markov chain whose state carries over from frame to frame; the burst model flips a run of L bits. Everything draws from one seeded random number generator, so a run can always be repeated.

How we evaluate: goodput is the number of correct frames times 128 payload bits, divided by every coded bit sent, so parity bits and retransmissions count against it. Latency comes from a simulated clock with an RTT of 20 ms, a timeout of 100 ms, at most 4 retries and a 160 kbit/s link. These values are our assumptions.

# 5. Database / UML / DFD

The system does not use a relational database. Everything it stores is in files: a per-frame CSV or JSON-lines log (D1 in the data-flow diagram) and the benchmark CSVs in results/. The diagrams below show how data moves through the system and how the software is organised.

Figure 5. Level-1 data-flow diagram.

Figure 6. Component/class diagram of the shared library.

### Log record (per frame)

Table 5. Per-frame log schema (tollgate.csv / --jsonl)

| Field | Meaning |
| --- | --- |
| seq, code, attempt | frame number, code used, transmission attempt |
| flipped, corrected | bits flipped by the channel / corrected by the decoder |
| crc_ok, class | CRC result and CORRECT / DETECTED_FAIL / SILENT_WRONG |
| decode_us | decode latency in microseconds |

# 6. Module Development

Table 6. Library modules (19 at v0.1, 26 at v0.2)

| Module(s) | Responsibility |
| --- | --- |
| bits, crc32, rng, gf | bit utilities, CRC-32 (IEEE 802.3), seeded RNG, GF(2^m) arithmetic |
| hamming, secded, bch, rs, ldpc, interleave | the error-correcting codes and the bit block interleaver |
| fec | single registry: choose a code, encode, decode, rate, description |
| frame, packet | payload ID96 ¦¦ CRC32; wire header (v2, 16 bytes), parsing and validation |
| noise, profiles, schedule | BSC / Gilbert-Elliott / burst, the two profiles, toll->hospital->toll schedule |
| receiver | decode, CRC check, classification (never reads ground truth) |
| sender, adapt, window, feedback | ARQ/HARQ state machine, adaptive ladder controller, sliding-window statistics, ACK/NAK wire format |
| sim, linksim, stats | in-process link simulator on a simulated clock, goodput/latency, Student-t 95% CIs |
| apps/vehicle, channel, tollgate | the three UDP processes |
| tools/bench, plot.py, dashboard.py | benchmark, plots, live dashboard |

We worked in phases and wrote the tests first. A phase was only committed once `make test` passed.

# 7. Working Prototype

The prototype runs on Linux, WSL and macOS. All you need is a C compiler and make; Python 3 is only needed for the dashboard and the plots.

git clone https://github.com/PavitranVC/Adaptive-FEC-Link.git && cd Adaptive-FEC-Link
make                 # build vehicle, channel, tollgate, bench
make test            # unit + integration tests
make demo            # one-terminal live demo (toll plaza)
make demo-hospital   # bursty hospital profile
make demo-adaptive   # adaptive FEC on toll -> hospital -> toll
make dashboard       # live web dashboard on http://127.0.0.1:8050

In a demo of 20 frames on the toll profile with Hamming(7,4), all 20 frames came out CORRECT, with no DETECTED_FAIL and no SILENT_WRONG. The channel flipped 48 of 4,480 bits (a measured BER of 0.0107 against the model's 0.0100) and the decoder fixed all 48. The tollgate also prints a coloured bit map for each frame that shows which bits were flipped and which were corrected.

# 8. Implementation Progress

Table 7. Implementation progress

| Phase / version | Deliverable | Status |
| --- | --- | --- |
| Phase 0 | scaffolding, Makefile, folder layout, docs | Done |
| Phase 1 | bit utils, CRC-32, Hamming(7,4)/(15,11), SECDED(8,4) | Done |
| Phase 2 | BSC, Gilbert-Elliott, burst noise, two profiles | Done |
| Phase 3 | UDP pipeline: vehicle, channel, tollgate, CLI, logs, demos | Done |
| Phase 4 | BCH(15,7), BCH(31,16): GF(2^m), Berlekamp-Massey, Chien | Done |
| Phase 5 | in-process benchmark and plots | Done |
| Phase 6 | Reed-Solomon over GF(256) + generic interleaver | Done |
| v0.2 | ground-truth integrity check, Stop-and-Wait ARQ, HARQ, adaptive controller, dashboard | Done |
| v0.3 (planned) | fair baselines (always-strongest code), multi-seed statistics, failure injection, sanitizers | Next |
| v0.4 (planned) | continuous integration, report, optional LDPC stretch | Next |

At v0.2 the project has 26 library modules, 20 unit-test programs with 123 test cases, an integration script that runs real three-process UDP scenarios, 12 plots and 4 benchmark CSV files.

# 9. Test Cases

We wrote each test before the code it checks. `make test` builds and runs all of them, and everything passed at the time of this review.

Table 8. Test cases (v0.2)

| ID | Area | Test | Expected result |
| --- | --- | --- | --- |
| T1 | bits / CRC-32 | known vectors; single, double and triple bit errors on 128-bit messages | every 1-, 2-, 3-bit error detected |
| T2 | Hamming | every single-bit error at every position | corrected |
| T3 | Hamming + CRC | 2-bit error on plain Hamming | miscorrects; CRC-32 catches it |
| T4 | SECDED | 2-bit errors | detected, not miscorrected |
| T5 | noise | measured flip rate / burst length vs parameter | statistically close to model |
| T6 | BCH(15,7) | every error pattern of weight <= 2 (exhaustive) | corrected |
| T7 | BCH(31,16) | randomized patterns of weight <= 3 | corrected |
| T8 | Reed-Solomon | up to t byte errors and bursts; interleaver round trip | corrected |
| T9 | receiver integrity | ground truth stripped from the packet | classification unchanged (decoder never reads truth) |
| T10 | feedback / sender | ACK, NAK, timeout, retry limit | correct state transitions |
| T11 | adaptive controller | step up on bursts, step down on calm, no flapping | level follows channel, no flapping |
| T12 | integration | real UDP runs: reproducibility by seed, hospital profile, BCH, RS, interleaver, ARQ, HARQ with 30% feedback loss, adaptive schedule | all scenarios pass |
| T13 | dashboard | JSON-lines log -> state over HTTP | correct counts |

# 10. Initial Results

These numbers use a single seed (42) and 5,000 frames per point. GE means Gilbert-Elliott with p_gb = 0.005, and the noise parameters are our own assumptions. Every value is a frame success rate.

Table 9. Initial code comparison (benchmark E1)

| code | rate | toll BSC p=0.01 | toll GE | hospital GE | burst L=16 |
| --- | --- | --- | --- | --- | --- |
| uncoded | 1.0000 | 0.2700 | 0.3328 | 0.3952 | 0.0000 |
| Hamming(7,4) | 0.5714 | 0.9354 | 0.7752 | 0.4170 | 0.0014 |
| Hamming(7,4) + il8 | 0.5714 | 0.9360 | 0.9394 | 0.6106 | 0.0956 |
| Hamming(15,11) | 0.7111 | 0.8956 | 0.7822 | 0.4720 | 0.0004 |
| SECDED(8,4) | 0.5000 | 0.9174 | 0.7406 | 0.3654 | 0.0008 |
| BCH(15,7) | 0.4491 | 0.9932 | 0.8978 | 0.3912 | 0.0044 |
| BCH(31,16) | 0.5161 | 0.9986 | 0.9596 | 0.4936 | 0.0126 |
| BCH(31,16) + il8 | 0.5161 | 0.9980 | 0.9978 | 0.8374 | 1.0000 |
| RS t=4 | 0.6667 | 0.9634 | 0.9756 | 0.8336 | 1.0000 |

Figure 7. Frame success rate vs BSC bit-error probability.

Figure 8. Frame success rate vs burst length: bit-level codes collapse, Reed-Solomon and interleaving survive.

### Initial strategy comparison (toll -> hospital -> toll, seed 42)

Table 10. Strategies at v0.2

| strategy | success | retx / frame | p99 latency (ms) |
| --- | --- | --- | --- |
| Stop-and-Wait ARQ (uncoded) | 0.8194 | 1.7828 | 94.009 |
| HARQ BCH(31,16) | 0.9940 | 0.1980 | 76.226 |
| Adaptive (mixed ladder) | 0.9994 | 0.0764 | 33.211 |

Figure 9. Adaptive level over time: the controller reacts within about 80 frames to the hospital burst and does not flap.

### Observations

BCH(31,16) does best on random errors (0.9986 at p = 0.01), while bursts need Reed-Solomon or an interleaver.

Out of 1,395,000 frames per profile, none was SILENT_WRONG. The CRC-32 caught every miscorrection.

The adaptive scheme brings retransmissions down from 1.78 per frame (plain ARQ) to 0.076, and p99 latency from 94 ms to 33 ms.

One thing we have not checked yet is whether the adaptive controller really beats simply always using the strongest code, and all of this comes from one seed. That is the first job after this review.

Keep in mind that these results come from one seed, a simulated channel with assumed parameters and no hardware.

# 11. Individual Contribution

To be completed by the team before upload. The repository history (`git log --author`) can back this up.

Table 11. Individual contributions

| Member | Reg. No. | Contribution (this review) |
| --- | --- | --- |
| [Your Name] | [Reg. No.] | All coding and implementation: system architecture; CRC-32, bit utilities, seeded RNG and GF(2^m) arithmetic; Hamming, SECDED, BCH and Reed-Solomon codes and the interleaver; BSC, Gilbert-Elliott and burst noise models and the two profiles; the vehicle, channel and tollgate UDP programs; Stop-and-Wait ARQ, HARQ and the adaptive controller; the benchmark, plots and dashboard; unit, integration and failure tests. |
| [Teammate Name] | [Reg. No.] | Documentation and demo preparation: report writing, diagrams and figure captions, test-case documentation, collecting and checking results, demo scripts and rehearsal. |

# 12. Plan for Review 3 and References

Compare the adaptive controller against HARQ with a fixed RS t=4 and RS t=8, using goodput as the main measure.

Repeat the experiments over 20 seeds and report 95% confidence intervals and paired differences, plus a test of how RTT changes the result.

Try to break the system on purpose with malformed, truncated and duplicate packets, a lost END message and a busy port, and run everything under ASan and UBSan.

Set up continuous integration, write the final report, attempt LDPC as an extra, and prepare the live demonstration and the comparison with the existing approach.

References

[1] S. Lin and D. J. Costello, Error Control Coding, 2nd ed. Upper Saddle River, NJ: Pearson Prentice Hall, 2004.

[2] R. E. Blahut, Algebraic Codes for Data Transmission. Cambridge, UK: Cambridge University Press, 2003.

[3] E. N. Gilbert, "Capacity of a burst-noise channel," Bell Syst. Tech. J., vol. 39, no. 5, pp. 1253-1265, Sept. 1960.

[4] E. O. Elliott, "Estimates of error rates for codes on burst-noise channels," Bell Syst. Tech. J., vol. 42, no. 5, pp. 1977-1997, Sept. 1963.

[5] IEEE Std 802.3, IEEE Standard for Ethernet (CRC-32 frame check sequence).

Supporting material in the project repository: docs/DESIGN_DECISIONS.md, docs/ALGORITHMS.md.